NAME- SIDDHESH DILIP KHAIRNAR

ROLL NO- 1421

DIVISION- N

PRN NO- 22110398

BEE TERM WORK

### EXPERIMENT NO.1

Study of safety precautions and measuring instruments

1.6 Study of resistors, capacitors and inductors

| Sr. No. | Color Code | Tolerance (%) | Resistance Value (KΩ) |
| --- | --- | --- | --- |
| 1 | Red, Black, Yellow, Gold | 5% | 200 KΩ |
| 2 | Brown, Black, Orange, Gold | 5% | 10KΩ |
| 3 | Yellow, Violet, Red, Silver | 10% | 4.7KΩ |

### EXPERIMENT NO.3

### Verification of Kirchhoff’s Laws and Superposition Theorem

  - Result Table:

| Sr. No. | E1 (volts) | E2 (volts) | I1 (mA) | V1 (V) | I2 (mA) | V2 (V) | I3 (mA) | V3 (V) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 21 | 0 | 252 | 12.6 | 168 | 8.4 | 84 | 8.4 |
| 2 | 0 | 24 | 192 | 9.6 | 288 | 14.4 | 96 | 9.6 |
| 3 | 21 | 24 | 60 | 3 | -120 | -6 | 180 | 18 |

  - Verification Table:

- For calculation 1 i.e. When only E1 is operative, if I1= I2+I3, KCL is verified. Similarly, if E1= V1+V3 then KVL is verified.

- For calculation 2 i.e. When only E2 is operative, if I2= I1+I3, KCL is verified. Similarly, if E2= V2+V3 then KVL is verified.

- For calculation 3, if I3 (with E1 only applied) + I3 (with E2 only applied) = I3 (with both E1 and E2 applied), then Superposition theorem is verified.

• Write the above-mentioned steps in the verification table by putting respective values of currents and voltages for each calculation and complete the verification table.

| Calculation No. | Verification of KCL | Verification of KVL |
| --- | --- | --- |
| 1 | 192 + 60 = 252 | 9.6 + 3 = 12.6 |
| 2 | 288 – 120 = 168 | 14.4 – 6 = 8.4 |
| 3 | ¦ 252 – 168 ¦ = 84 ¦ 192 – 288 ¦ = 96 ¦ 60 + 120 ¦ = 180 |  |

### EXPERIMENT NO.4

### Study of single-phase R-C series circuit

  - Given Parameters:

| Sr. No. | V (volts) | I (A) | VR (V) | VC (V) |
| --- | --- | --- | --- | --- |
| 1 | 200 | 0.23 | 42 | 184 |

  - Calculations:

Give appropriate units, as applicable, to different quantities calculated.

    - Resistance of circuit= R=VR/ I = 182.6 Ω

    - Capacitive reactance=XC= VC/ I = 800 Ω

    - With frequency f= 50 Hz, Capacitance of circuit=C=1/2π f XC = 3.9 μF

    - Circuit impedance=Z=V/ I = 869.56 Ω

    - Power factor of the circuit= cos ø= R/Z = 0.2099 rad leading

4.10 Phasor Diagram:

4.11 Results:

    - Resistance of the circuit = R= 182.6 Ω

    - Capacitance of the circuit = C = 3.9 μF

    - Power factor of the circuit = cos ø= 0.2099 rad

EXPERIMENT NO. 5

### Direct Loading Test on Single Phase Transformer

  - Given Parameters:

| Sr.No. | V1 (V) | I1 (A) | W1 (W) | V2 (V) | I2 (A) | W2 (W) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 110 | 0.22 | 20*2 | 216 | 0 | 0 |
| 2 | 110 | 2 | 115*2 | 214 | 0.8 | 80*2 |
| 3 | 110 | 3.6 | 220*2 | 210 | 1.7 | 150*2 |
| 4 | 110 | 5.2 | 320*2 | 200 | 2.5 | 260*2 |
| 5 | 110 | 6.75 | 430*2 | 198 | 3.3 | 340*2 |
| 6 | 110 | 8.2 | 525*2 | 192 | 4.15 | 410*2 |

  - Calculations:

• From circuit diagram, power rating = Q = 1 kVA, rated primary voltage= V1= 110 V, Rated secondary voltage = V2 = 220 V

- Voltage ratio = V1/V2 = 0.5

- I1 (rated) = (Q × 103)/ (rated V1) = 9.09 A

- I2 (rated) = (Q ×103)/ (rated V2) = 4.54 A

- Current ratio = I1/I2 = 2

- Percentage Efficiency = [W2/ W1] ×100

- % Regulation = [(E2 – V2)/ E2] ×100 Where, E2 = Secondary voltage on no load, V2 = Secondary voltage on load

(Note: value of E2 will be same i.e., 220 V for all calculations)

5.8 Result Table:

- Voltage ratio =    0.5

- Current ratio = 2

| Sr. No. | Output Current I2 (A) | Output Power W2 (W) | Efficiency (%) | Voltage Regulation (%) |
| --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 1.82 |
| 2 | 0.8 | 160 | 69.57 | 2.73 |
| 3 | 1.7 | 300 | 68.18 | 4.55 |
| 4 | 2.5 | 520 | 81.25 | 9.09 |
| 5 | 3.3 | 680 | 79.07 | 10 |
| 6 | 4.15 | 820 | 78.1 | 12.7 |

5.9 Graphs:

Draw following graphs by selecting appropriate scales.

- % Efficiency (on Y-axis) Vs. Power Output (on X-axis)

- % Voltage Regulation (on Y-axis) Vs. Output Current (on X-axis)

EXPERIMENT NO.6

Verification of Voltage and Current Relations and Power Calculations in Three Phase Star and Delta connected balanced Loads

  - Given Parameters:

For Star connection

| Sr. No. | Line Voltage (V) | Phase Voltage (V) | Line Current (A) | Phase Current (A) |
| --- | --- | --- | --- | --- |
| 1 | 354 | 200 | 0.89 | 0.89 |
| 2 | 392 | 224 | 0.93 | 0.93 |
| 3 | 440 | 245 | 1 | 1 |

For Delta connection

| Sr. No. | Line Voltage (V) | Phase Voltage (V) | Line Current (A) | Phase Current (A) |
| --- | --- | --- | --- | --- |
| 1 | 448 | 448 | 2.2 | 1.3 |
| 2 | 448 | 448 | 2.9 | 1.7 |
| 3 | 448 | 448 | 3.7 | 2.1 |

  - Calculations:

For star connection:

VL = √3 VPH [calculate line voltage for each reading by using this formula and phase voltage reading]

- VL1 = √ 3VPH1 = 346.410 V, Ptotal = √3 VL1 IL1 cosø = 533.99 W

- VL2 = √ 3VPH2 = 387.979 V, Ptotal = √3 VL2 IL2 cosø = 624.95 W

- VL3= √ 3VPH3 = 424.352 V, Ptotal = √3 VL3 IL3 cosø = 734.99 W

For delta connection:

IL=√3 IPH [calculate line current for each reading by using this formula and phase current reading]

- IL1 = √ 3IPH1 = 2.25 A, Ptotal = √3 VL1 IL1 cosø = 1745.9 W

- IL2 = √ 3IPH2 = 2.94 A, Ptotal = √3 VL2 IL2 cosø = 2281.31 W

- IL3 = √ 3IPH3 = 3.63 A, Ptotal = √3 VL3 IL3 cosø = 2816.73 W

6.8 Result Table:

| Sr. No. | Line Voltage Given Star (V) | for | Line Voltage Calculated for Star (V) | Line Current Given for Delta (A) | Line Current Calculated for Delta (A) | Total Power Consumed by Star Load (W) | Total Power Consumed by Delta Load (W) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 354 |  | 346.41 | 2.2 | 2.25 | 533.99 W | 1745.9 W |
| 2 | 392 |  | 387.97 | 2.9 | 2.94 | 624.95 W | 2281.31 W |
| 3 | 440 |  | 424.35 | 3.7 | 3.63 | 734.99 W | 2816.73 W |

- 9 Phasor Diagrams:

  - Phasor diagram for star

  - Phasor diagram for delta